Winter Wx Flying

Procedures and Tips

I.	AIRCRAFT PREPARATION:

- Baffles — Intake & oil cooler covers.

- Engine Oil - SAE IOW30 oil is recommended for below 40F. — improves cold starts. (POH).

- Oil Breather - Crankcase breather line freezing causes hi-pressure/ruptured lines and/or blown oil

caps.

4.	Hose clamps, hoses, hydraulic fittings & seals — loose and/or deteriation

5	 Cabin Heater — heater system shroud lines — tight and sealed; CO monitor in cabin area.

- Control Cables — Proper adjustment(s) and lube — cold cause constrictions.

- Oil Pressure Controlled Props — oil congeals — allow to warm up, and cycle the prop.

- Batteries — Keep warm (or remove) and fully charged — Aircraft/flashlight and ELT.

- Wheel Wells and Wheel Pants — Keep slush free; Remove wheel pants.

II.	PREFLIGHT OPERATIONS:

- Fuel Contamination — Keep the fuel tanks full to lessen condensation

- Fueling Facilities — Be wary of above ground tanks; look for water filters/separators.

3	   Aircraft Fuel Sumps — Internal lines, filters and sumps can freeze — always drain the sump!

- Aircraft Preheat — (1) Park in a heated hangar, (2) use blankets in engine compartment and cockpit area;

(3) Use a preheater (electric and/or ducted).

- Ice, Snow, Frost — Remove all surface ice/snow/frost; Drain all water from inside/outside surfaces.

- Blowing Snow — shield aircraft from blowing snow/precipitation; Can clog critical instruments and flight controls, even refreezing during takeoff.

- Fuel Vents — Checked unplugged; can cause fuel tanks/bladders to rupture or collapse, or engine stoppage.

- Parking — Do NOT set the brakes as moisture/ice buildup may freeze the brakes. (POH)

III. 	ENGINE STARTS:

- Preheat - Use electric or ducted when available.

- Priming — Tendency to over-prime which washes down the cylinder walls, lowers compression, scours the walls, and makes for harder starts, backfires or fires

- Idling — Avoid prolonged idling — the engine can quite due to iced over and fouled plugs.

- Stabilized Engine — Allow engine compartment to warm up prior to turning on electrical system and Radios.

IV.	BEFORE TAKEOFF:

1	Flight Controls - Smooth operation; drained free of water.

- Taxiing - Avoid short turns or quick stops; keep clear of slush (clogs/freezes brakes).

- Skis - Any Skis?? Again, avoid sharp turns and sudden stops (adverse torque on the Idg gear).

- Takeoff Computations — Takeoff ground run will INCREASE ! ! !

5	Heated Instruments — Turn on Pitot Heat, etc.; Ensure instruments/radios are warmed, operate freely.

V. 	TAKEOFF:

- Flight into known icing conditions is prohibited. (POH)

- Engine Overboost - Monitor the engine manifold, rpm, temp gauges closely (Power output increases about 1% per 10 deg. F below standard. At 40 deg. F, the engine will develop 10% greater HP even tho MP and Temps are not exceeded).

3.	Carb Heat — Use prior and during takeoff— adjust mixture as required.

4.	Heated Instruments — Turn on Pitot tube, etc., heaters, preferably on taxi out.

5.	On Climb Out — Closely monitor engine instruments and head temps.